# CHAPTER THREE: METHODOLOGY

## 3.1 Research Design

This research employs a quantitative, applied research design combining conventional geophysical interpretation methods with computational approaches. The workflow is structured as a sequential pipeline where outputs from earlier stages serve as inputs to subsequent analyses, ensuring internal consistency and traceability.

The research philosophy is grounded in the principle that modern seismic interpretation should be: - Quantitative: All interpretations supported by measurable metrics - Reproducible: Methods documented such that results can be replicated - Calibrated: Seismic observations tied to well control wherever possible - Comprehensive: Full utilization of available data coverage

## 3.2 Data Description

### 3.2.1 3D Seismic Data

The primary dataset comprises a 3D post-stack seismic survey covering the central Bornu Chad Basin:

| Parameter | Value |
| --- | --- |
| Survey area | ~1,955 km² |
| Inline range | 5047 - 6047 |
| Crossline range | 4885 - 7020 |
| Total traces | 1,844,191 |
| Sample interval | 4 ms |
| Record length | 8,000 ms |
| Data format | SEG-Y |
| Processing state | Post-stack, migrated |

The data was acquired in the 1980s and processed using conventional workflows of that era. Expected characteristics include limited bandwidth (8-15 Hz dominant frequency) and moderate signal-to-noise ratio.

### 3.2.2 2D Seismic Data

Regional context will be provided by 62 2D seismic lines from the CH-78 survey:

| Parameter | Value |
| --- | --- |
| Survey designation | CH-78 |
| Acquisition year | 1978 |
| Total lines | 62 |
| Processing types | STK (stacked), MIG (migrated) |
| Dominant frequency | ~18 Hz |

### 3.2.3 Well Log Data

Six wells with comprehensive log suites will provide subsurface calibration:

| Well | Location Status | Available Logs | Depth Range |
| --- | --- | --- | --- |
| BULTE-1 | Within 3D | GR, DT, RHOB, RES | 43 - 1,465 m |
| HERWA-01 | Within 3D | GR, DT, RHOB, RES | 322 - 4,705 m |
| KASADE-01 | Within 3D | GR, DT, RHOB, RES | 455 - 1,600 m |
| MASU-1 | Outside 3D | GR, DT, RHOB, RES | 1,996 - 3,105 m |
| NGAMMAEAST-1 | Outside 3D | GR, DT, RHOB, RES | 401 - 3,232 m |
| NGORNORTH-1 | Outside 3D | GR, DT, RHOB, RES | 457 - 657 m |

Log mnemonics: GR = Gamma Ray, DT = Sonic (transit time), RHOB = Bulk Density, RES = Resistivity

## 3.3 Data Quality Assessment

### 3.3.1 Seismic Quality Control

The first phase of work involves systematic quality assessment of the seismic data:

Amplitude Statistics: - Calculate mean, standard deviation, minimum, maximum amplitudes - Generate amplitude histograms to assess distribution - Identify amplitude anomalies (null traces, spikes)

Dead Trace Analysis: - Identify traces with zero or near-zero variance - Calculate percentage of dead/unusable traces - Assess spatial distribution of data gaps

Frequency Analysis: - Compute amplitude spectra using Fast Fourier Transform (FFT) - Determine dominant frequency and bandwidth - Calculate vertical resolution using λ/4 criterion:

Resolution = V / (4 × f)

Where V = average velocity, f = dominant frequency

Signal-to-Noise Ratio (SNR): - Estimate SNR using coherence-based methods - Assess spatial variation in data quality

### 3.3.2 Well Log Quality Control

Well logs will be assessed for: - Completeness of log suites - Presence of bad hole flags (caliper log) - Consistency between related measurements - Depth registration accuracy

Quality tiers will be assigned: - Tier 1: Complete logs, good hole conditions - suitable for all analyses - Tier 2: Partial logs or minor issues - suitable for selected analyses - Tier 3: Significant gaps or quality issues - limited use

## 3.4 Well Log Analysis and Petrophysics

### 3.4.1 Log Conditioning

Raw logs will be conditioned through: - Environmental corrections (if required) - Depth matching between curves - Despiking and filtering of noise - Null value handling

### 3.4.2 Petrophysical Calculations

Shale Volume (Vsh): Using the linear gamma ray method:

Vsh = (GR_log - GR_clean) / (GR_shale - GR_clean)

Where: - GR_log = measured gamma ray value - GR_clean = gamma ray in clean sand (minimum) - GR_shale = gamma ray in shale (maximum)

Porosity (φ): From density log:

φ_density = (ρ_ma - ρ_b) / (ρ_ma - ρ_f)

Where: - ρ_ma = matrix density (2.65 g/cc for sandstone) - ρ_b = bulk density from log - ρ_f = fluid density (1.0 g/cc for water-based mud)

Water Saturation (Sw): Using Archie’s equation:

Sw = [(a × Rw) / (φ^m × Rt)]^(1/n)

Parameters: - a = tortuosity factor = 1.0 - m = cementation exponent = 2.0 - n = saturation exponent = 2.0 - Rw = formation water resistivity (estimated from clean water zones) - Rt = true resistivity from deep reading tool

Velocity Calculation: From sonic log:

V = 304,800 / DT

Where: - V = velocity (m/s) - DT = transit time (μs/ft) - standard LAS units

### 3.4.3 Fluid Typing

Hydrocarbon zones will be identified using: - Sw < 50% criterion for pay - Resistivity-porosity crossplots - Density-neutron separation (gas effect) - Fluid contact identification from Sw profiles

## 3.5 Well-to-Seismic Tie

### 3.5.1 Synthetic Seismogram Generation

The convolution model will be applied:

Synthetic = Wavelet * Reflection Coefficient Series

Step 1: Calculate Acoustic Impedance (AI)

AI = V × ρ

Where V = velocity from sonic, ρ = density from RHOB log

Step 2: Calculate Reflection Coefficients (RC)

RC_i = (AI_{i+1} - AI_i) / (AI_{i+1} + AI_i)

Step 3: Extract/Generate Wavelet

A Ricker wavelet matched to the seismic dominant frequency (estimated 12 Hz) will be used initially. Statistical wavelet extraction from seismic will be attempted where data quality permits.

Step 4: Convolution

Convolve RC series with wavelet to produce synthetic trace.

### 3.5.2 Correlation Optimization

The synthetic will be correlated with extracted seismic traces at well locations:

Correlation Coefficient (r):

r = Σ[(Si - S̄)(Xi - X̄)] / √[Σ(Si - S̄)² × Σ(Xi - X̄)²]

Where: - Si = synthetic sample values - Xi = seismic sample values - S̄, X̄ = mean values

Optimization Parameters: - Time shift: ±200 ms search range - Phase rotation: 0° to 180° in 10° increments

The combination yielding maximum correlation will define the optimal tie.

Quality Classification: - r > 0.7: EXCELLENT - r = 0.5-0.7: GOOD - r = 0.3-0.5: FAIR - r < 0.3: POOR

## 3.6 Horizon Interpretation

### 3.6.1 Target Horizons

Four horizons corresponding to major formation tops will be interpreted:

| Horizon | Seismic Character | Approximate TWT |
| --- | --- | --- |
| Top Chad Formation | Weak, discontinuous | 400-600 ms |
| Top Fika Shale | Trough (negative) | 900-1100 ms |
| Top Gongila Formation | Trough (negative) | 1100-1300 ms |
| Top Bima Sandstone | Peak (positive) | 1500-1700 ms |

### 3.6.2 Interpretation Workflow

Step 1: Seed Point Definition - Identify horizon times at well locations from synthetic ties - Create seed points at each well with calibrated TWT picks

Step 2: Auto-Tracking - Propagate picks from seeds using amplitude-guided tracking - Search window: ±100 ms around seed/adjacent pick - Track mode: Peak for Top Bima, Trough for others

Step 3: Quality Control - Calculate coverage percentage - Identify and manually correct mis-picks - Apply geological constraints (horizon cannot cross)

Step 4: Gridding and Mapping - Grid picks to regular grid using minimum curvature - Apply smoothing filter (Gaussian, σ=2) - Generate time-structure maps

### 3.6.3 Deliverables

For each horizon: - Time-structure map with contours - Coverage statistics - TWT range and relief

## 3.7 Seismic Attribute Analysis

### 3.7.1 Horizon-Based Attributes

RMS Amplitude will be extracted in a window around each horizon:

RMS = √(Σ A_i² / n)

Window: ±50 ms centered on horizon pick

Anomaly Detection: - Calculate mean and standard deviation of attribute - Flag points exceeding 2σ as potential DHIs - Map spatial distribution of anomalies

### 3.7.2 Volume Attributes

Variance Attribute for fault detection:

Calculated in a 3×3 trace window at specified time slice:

Var = Σ(A_i - Ā)² / (n-1)

High variance indicates trace-to-trace discontinuity associated with faults.

## 3.8 Fault Interpretation

### 3.8.1 Fault Detection

Faults will be identified through:

Method 1: Horizon Gradient Analysis - Calculate gradient magnitude of horizon surface - Threshold high gradients as fault indicators

Method 2: Variance Attribute - Generate variance time slices at reservoir levels - Identify linear high-variance features as faults

### 3.8.2 Fault Characterization

For each identified fault: - Throw: Vertical displacement calculated from horizon offset - Strike: Orientation from fault trace azimuth - Length: Extent of continuous fault trace - Significance: Ranked by throw × length product

### 3.8.3 Fault Filtering

To focus on regionally significant faults: - Minimum length threshold: 100 data points - Minimum throw threshold: 10 ms - Top 50 faults reported and characterized

## 3.9 Structural Analysis and Closure Identification

### 3.9.1 Closure Detection

Structural closures will be identified from time-structure maps:

Algorithm: 1. Identify local minima (structural highs in TWT) 2. Expand contours from crest until spillpoint reached 3. Calculate closure relief and area 4. Filter by minimum criteria (relief > 20 ms, area > 5 km²)

### 3.9.2 Trap Classification

Traps will be classified as: - Four-way dip closure - Three-way dip against fault - Fault-bounded trap - Stratigraphic (if identified from amplitude)

## 3.10 Volumetric Estimation

### 3.10.1 Deterministic Estimation

STOIIP calculated using:

STOIIP (STB) = (7,758 × A × h × φ × (1-Sw) × NTG) / Bo

Where: - A = area (acres) - h = net pay (feet) - φ = porosity (fraction) - Sw = water saturation (fraction) - NTG = net-to-gross ratio - Bo = formation volume factor

### 3.10.2 Probabilistic Estimation

Monte Carlo simulation with 10,000 iterations using triangular distributions:

| Parameter | Min | Mode | Max |
| --- | --- | --- | --- |
| Area (km²) | 10 | 15 | 20 |
| Net pay (m) | 30 | 50 | 70 |
| Porosity | 0.15 | 0.20 | 0.25 |
| Sw | 0.20 | 0.25 | 0.35 |
| Bo | 1.1 | 1.2 | 1.3 |
| NTG | 0.5 | 0.6 | 0.7 |

Output: P10, P50, P90 resource estimates

## 3.11 LLM-Assisted Interpretation Framework

A key innovation of this research is the development of an LLM-assisted interpretation workflow, implemented through a custom graphical user interface (GUI). This framework represents a paradigm shift from traditional interpretation software.

### 3.11.1 System Architecture

The interpretation framework consists of three integrated components:

┌─────────────────────────────────────────────────────────────────┐
│              LLM-ASSISTED INTERPRETATION FRAMEWORK              │
├─────────────────────────────────────────────────────────────────┤
│                                                                 │
│  ┌─────────────────┐   ┌─────────────────┐   ┌──────────────┐  │
│  │   DATA LAYER    │   │ PROCESSING LAYER │   │  LLM LAYER   │  │
│  │                 │   │                 │   │              │  │
│  │ • SEGY Reader   │──▶│ • Well Analysis │──▶│ • Result     │  │
│  │ • LAS Reader    │   │ • Horizon Track │   │   Analysis   │  │
│  │ • Data QC       │   │ • Attributes    │   │ • Visual     │  │
│  │ • Velocity Model│   │ • Fault Detect  │   │   Interpret  │  │
│  │                 │   │ • Volumetrics   │   │ • Decision   │  │
│  │                 │   │                 │   │   Guidance   │  │
│  └─────────────────┘   └─────────────────┘   └──────────────┘  │
│           │                    │                    │          │
│           └────────────────────┼────────────────────┘          │
│                                ▼                               │
│                    ┌─────────────────┐                         │
│                    │  GUI INTERFACE  │                         │
│                    │                 │                         │
│                    │ • Interactive   │                         │
│                    │   Displays      │                         │
│                    │ • User Controls │                         │
│                    │ • LLM Chat      │                         │
│                    │ • Report Gen    │                         │
│                    └─────────────────┘                         │
│                                                                 │
└─────────────────────────────────────────────────────────────────┘

### 3.11.2 LLM Integration Capabilities

The Large Language Model component provides:

Result Interpretation: - Analyzes numerical outputs (correlation coefficients, amplitude statistics, fault parameters) - Provides geological context for quantitative results - Identifies anomalies and patterns requiring attention - Generates natural language summaries of interpretation products

Visual Analysis: - Interprets generated maps, cross-sections, and attribute displays - Identifies geological features (faults, closures, amplitude anomalies) - Compares visual outputs with expected geological patterns - Flags potential interpretation issues or artifacts

Decision Guidance: - Recommends next steps based on current results - Suggests parameter adjustments for improved results - Identifies gaps in interpretation coverage - Prioritizes interpretation tasks based on data quality and objectives

Documentation: - Automatically generates interpretation reports - Documents decisions and rationale - Maintains audit trail of interpretation workflow - Creates presentation-ready summaries

### 3.11.3 GUI Implementation

The graphical user interface is implemented using Python’s Tkinter framework, providing:

| Feature | Description |
| --- | --- |
| Data Browser | Navigate seismic volumes, wells, and interpretation products |
| Inline/Crossline Viewer | Interactive seismic section display with horizon overlays |
| Map Display | Time-structure and attribute maps with fault overlays |
| Well Correlation | Multi-well displays with synthetic seismograms |
| LLM Chat Interface | Natural language interaction for queries and guidance |
| Workflow Control | Step-by-step interpretation with progress tracking |
| Report Generator | Automated creation of interpretation documents |

### 3.11.4 Workflow Automation

The framework automates the interpretation workflow while maintaining user control:

- Data Loading: Automatic format detection, QC, and validation

- Well Analysis: Guided petrophysical calculation with quality feedback

- Well Ties: Automated correlation optimization with visual verification

- Horizon Tracking: Seed-based auto-tracking with coverage monitoring

- Attribute Analysis: Statistical anomaly detection with geological filtering

- Fault Interpretation: Automated detection with significance ranking

- Prospect Evaluation: Integrated analysis with resource estimation

- Reporting: Automated generation of interpretation deliverables

At each step, the LLM analyzes results and provides guidance for the next phase.

## 3.12 Software and Tools

| Task | Software/Tool |
| --- | --- |
| Seismic data handling | segyio (Python library) |
| Well log analysis | lasio (Python library) |
| Numerical computation | NumPy, SciPy |
| Visualization | Matplotlib, Seaborn |
| Machine learning | Scikit-learn |
| GPU acceleration | CuPy (NVIDIA CUDA) |
| Statistical analysis | SciPy.stats |
| GUI development | Tkinter (Python standard library) |
| LLM integration | Claude API (Anthropic) |
| Knowledge management | JSON-based knowledge base |

## 3.13 Workflow Summary

┌─────────────────────────────────────────────────────────────┐
│                    METHODOLOGY FLOWCHART                      │
├─────────────────────────────────────────────────────────────┤
│                                                               │
│  ┌─────────────┐    ┌─────────────┐    ┌─────────────┐      │
│  │   DATA QC   │───▶│  WELL LOG   │───▶│  WELL TIE   │      │
│  │  (Seismic)  │    │  ANALYSIS   │    │ CORRELATION │      │
│  └─────────────┘    └─────────────┘    └──────┬──────┘      │
│                                                │              │
│                                                ▼              │
│  ┌─────────────┐    ┌─────────────┐    ┌─────────────┐      │
│  │   FAULT     │◀───│  ATTRIBUTE  │◀───│  HORIZON    │      │
│  │ DETECTION   │    │  ANALYSIS   │    │ TRACKING    │      │
│  └──────┬──────┘    └─────────────┘    └─────────────┘      │
│         │                                                    │
│         ▼                                                    │
│  ┌─────────────┐    ┌─────────────┐    ┌─────────────┐      │
│  │  CLOSURE    │───▶│ VOLUMETRIC  │───▶│  PROSPECT   │      │
│  │   MAPPING   │    │ ESTIMATION  │    │  RANKING    │      │
│  └─────────────┘    └─────────────┘    └─────────────┘      │
│                                                               │
└─────────────────────────────────────────────────────────────┘

## 3.14 Quality Assurance

Quality will be assured through:

- Documentation: All parameters and decisions recorded

- Reproducibility: Workflows scripted for re-execution

- Calibration: All seismic interpretation tied to well control

- Peer review: Results reviewed against published knowledge

- Uncertainty quantification: Confidence ranges reported

- LLM verification: Automated consistency checks and anomaly detection

- Knowledge base validation: Results compared against established geological principles